Challenge: Multi-Core Scheduling of Computational Graphs under Limited Memory Bandwidth

Background

Traditional operator programming methods are difficult to generalize to large-scale model fusion structures due to difficulties in writing, limited support for fusion structures, and the challenge of balancing cost and generality. This creates a data communication barrier between operators during actual model execution. This creates a severe bandwidth bottleneck on memory-bandwidth-constrained edge platforms and limits the utilization of core computing resources. To address this issue, the PTO tool breaks down the entire computation process into a computational graph consisting of fine-grained atomic operation nodes, and fuses the entire network into a single operator. All operation nodes are scheduled offline, and an operator file that can be efficiently executed is generated. This breaks the data communication barrier between operators and yields significant performance and power-efficiency benefits in real-world service scenarios.

However, as edge NPU units evolve toward multi-core architectures, the computational graph scheduling algorithm in the current PTO faces a multi-core challenge: how to schedule an entire computational graph onto multiple cores, efficiently utilize each core's compute units and private cache, coordinate cross-core data dependencies, reduce DDR bandwidth occupation caused by inter-core data communication, and strike a balance among many constraints such as multi-core parallelism, on-core cache capacity limits, and shared main memory data transfer bandwidth, ultimately shortening the end-to-end execution time of the entire network on multiple cores. This challenge expects contestants to provide a multi-core scheduling algorithm based on static calendar scheduling to support the latest edge NPU platform in further improving compute unit utilization and energy efficiency.

Description

Schedule a directed acyclic graph (DAG) on a platform containing N homogeneous NPU cores and minimize the total simulation execution time. As shown in the figure, the computational graph is a bipartite graph consisting of tensor nodes and operation (op) nodes. Each tensor node contains two attributes: pos (whether the data is located in DDR or the on-core L1 cache) and size. Each op node contains two attributes: operation name (on-core/off-core data movement are named COPY_IN/COPY_OUT, and all other op nodes are compute nodes) and the execution pipe (pipe). The two types of op nodes adopt different time-cost models. For compute nodes, the execution time is a fixed value specified by the cycles attribute of the node. For data movement nodes, the execution time is dynamically calculated during scheduling simulation based on instantaneous bandwidth and the volume of data to be transferred.

Multi-core scheduling is implemented in two successive phases: (1) Graph partitioning: assign all compute nodes (excluding data movement nodes) of the complete computational graph into multiple subgraphs. No constraint is imposed on the number of subgraphs. (2) Subgraph scheduling: assign each subgraph to a specific core and determine the execution order of subgraphs on each core.

The algorithm shall output two sets of results: the graph partitioning scheme and the subgraph scheduling scheme. The sample evaluation program provided in the contest will supplement data movement operations across subgraph boundaries, determine the execution order of on-core operations, simulate multi-core parallel execution under shared bandwidth constraints based on the algorithm output, and finally produce the simulated execution time.

To satisfy physical constraints in multi-core scheduling, the sample evaluation program proceeds according to the following procedure.

(Notation conventions: ts(op) and tf(op) indicate the start time and end time of an op node; sg(op) and core(sg(op)) indicate the subgraph that the op node belongs to and the core where the op node is assigned; R(t) indicates the set of op nodes in execution at time t (where the set of data movement nodes is B(t)); size(op) indicates the data volume for a single data movement operation; L1(t) indicates the set of data residing in the on-core L1 cache at time t; W indicates the total DDR bandwidth shared by all cores.)

Step 1: Insert inter-core data copy nodes. For tensor nodes that require cross-core access after graph partitioning and scheduling, modify their location to DDR. Insert a COPY_OUT node into the producer-side subgraph and a COPY_IN node into the consumer-side subgraph.

Constraint 1: Inter-core data communication must go through DDR. That is, the cross-core data dependency path is u → COPY_OUT → DDR → COPY_IN → v.

Step 2: Determine on-core op execution order. Each core determines the execution order and dependencies of all its ops via a fixed heuristic pipeline: priority sorting → overflow control → local out-of-order scheduling. Priority sorting takes the scheduling sequence number of the subgraph to which the op node belongs as the primary key. Within the same subgraph, reverse DFS yields the order with the smallest cache occupancy. Therefore, the on-core execution order can be adjusted indirectly through graph partitioning and subgraph scheduling. Overflow control automatically inserts cache swap-in/swap-out nodes according to constraint 3. Local out-of-order scheduling determines the final execution order and memory reuse dependency edges based on the simulated occupancy of each pipe and cache. Contestants can also optimize the on-core execution order themselves, provided that both constraint 2 and constraint 3 are satisfied.

Constraint 2: The on-core execution order must satisfy the topological dependencies of the computational graph, that is, ts(v) ≥ tf(u), ∀(u,v) ∈ E (E is the set of dependency edges in the computational graph).

Constraint 3: During on-core scheduling execution, the volume of data residing in the on-core L1 cache at any time must satisfy τ ∈ L1(t)size(τ) ≤ CL1 (the L1 cache is 1 MB in size).

Constraint 4: At any time, only one op node can be in execution on each pipe within a core.

Step 3: Simulate multi-core execution. Simulate parallel multi-core execution using a discrete event loop. The initial time is t0 = 0. At each event time, retire finished ops, issue ops whose predecessors have all completed and whose pipe resources are idle, and advance the clock to the earliest finish time among all currently running ops as the next event time. When issuing data movement nodes, apply the principle of equal instantaneous bandwidth allocation: each data movement node in execution obtains a bandwidth of bi (t) =  W/|B(t)| , and the finish times of the remaining data movement ops in execution are postponed accordingly, so that the instantaneous bandwidth constraint is always satisfied. After all ops complete, output the simulated execution time.

Constraint 5: During simulated multi-core execution, the execution process of each core must meet constraints 2, 3, and 4.

Constraint 6: During simulated multi-core execution, the op execution order on each on-core pipe must strictly follow the execution order produced in step 2 for on-core scheduling. (There is no out-of-order execution mechanism in the pipe.)

Constraint 7: For ops with inter-core intermediate data dependencies, their execution times must also satisfy the topological dependencies of the computational graph. For an op node that receives cross-core data, it must wait 1,000 cycles after its producer op finishes execution before it can start execution (cross-core synchronization overhead).

Constraint 8: In the simulation results, the total volume of DDR data movement completed within any time interval must not exceed the interval length multiplied by the total bandwidth constant (the current total bandwidth is set to 60 bytes per cycle).

Test Cases

This challenge provides test cases for two basic structures, VGG16 and Matmul. Do not customize the test cases.

VGG16 network

The basic computational graph structure formed by the convolutional network is shown in the figure below. The input feature map is in [N, H, W, C] format. Each feature map is partitioned along the H axis, with a partitioning granularity of 2. Due to the computational characteristics of the convolution operator, the layers of the convolution computational graph exhibit an interleaved dependency structure. Therefore, the multi-core graph partitioning algorithm aims to obtain an optimal partitioning scheme to reduce the volume of inter-core communication data while decoupling inter-core dependencies as much as possible, thereby improving inter-core parallelism.

Matmul network structure

The computational graph formed by the Matmul network is shown in the figure below. The left and right matrices are loaded to the core in blocks for computation. In multi-core scheduling, the repeated data movement caused by multi-core computation must also be considered, as it can likewise create a bandwidth bottleneck: when the Matmul computation task is partitioned across multiple cores, one of the left and right matrices will undergo repeated data movement; as the number of cores increases, the latency may instead degrade due to excessive bandwidth consumption. Therefore, the multi-core scheduling algorithm aims to strike a balance between computation balance and bandwidth constraints, and maximize the degree of data reuse within a single core, avoiding performance degradation caused by data movement after core partitioning.

Criteria

Scoring is performed in the multi-core simulation environment, with a full score of 100 points, consisting of the following three metrics:

- Average multi-core speedup (40 points): S = 1Uu ∈ UTbase(u)Tsim(u), where U is the set of test cases. The speedup uses the single-core execution time of the test case as the baseline. The higher S, the better.

- Algorithm solution time (30 points): the time consumed by the contestant's algorithm to complete graph partitioning and subgraph scheduling for the test cases. The shorter, the better.

- Total DDR data movement volume (30 points): D = op ∈ COPYsize(op), where COPY refers to all COPY_IN/COPY_OUT nodes in the simulated multi-core execution. The smaller D, the better.